ROB – REV23
PILZ → SIEMENS PROFINET NON-SAFETY DIAGNOSTIC INTERFACE

Clarification / continuation of REV22 – Safety authority remains entirely in Pilz

# 1. Confirmed Engineering Clarification

The machine engineer has confirmed: “Non-safety data from the Pilz to the PLC is passed via PROFINET.”

Treat this as the architecture basis for the Pilz diagnostic interface. The Pilz safety controller remains responsible for the actual safety functions. The Siemens PLC receives NON-SAFETY status/diagnostic information over PROFINET for machine permissives, alarm diagnostics and HMI visualization.

# 2. Mandatory Architecture Boundary

- PILZ: executes all validated safety logic.

- PROFINET: transports non-safety diagnostic/status data between Pilz and Siemens PLC.

- SIEMENS PLC: consumes diagnostic/status data and exposes it to standard machine logic/HMI as required.

- WINCC UNIFIED HMI: displays Safety/Pilz diagnostics and alarms.

- No PROFINET diagnostic bit may be used as a substitute for a safety-rated Pilz function.

- No PLC/HMI command may bypass or force Pilz safety logic.

# 3. Required Pilz → PLC Diagnostic Dataset

Create a structured interface in the Siemens PLC for the diagnostic data exported by Pilz. Bind only signals that are proven in the final Pilz/PROFINET data map.

| Diagnostic Function | Expected Type | PROFINET Mapping | Status |
| --- | --- | --- | --- |
| Safety system ready/healthy | BOOL | VERIFY FROM PILZ DATA MAP | MISSING UNTIL VERIFIED |
| Pilz general fault | BOOL | VERIFY FROM PILZ DATA MAP | MISSING UNTIL VERIFIED |
| Safety reset required | BOOL | VERIFY FROM PILZ DATA MAP | MISSING UNTIL VERIFIED |
| Emergency-stop active – general | BOOL | VERIFY FROM PILZ DATA MAP | MISSING UNTIL VERIFIED |
| Individual E-stop status | BOOL array / individual BOOLs if exported | VERIFY FROM PILZ DATA MAP | MISSING UNTIL VERIFIED |
| Guard/door open – general | BOOL | VERIFY FROM PILZ DATA MAP | MISSING UNTIL VERIFIED |
| Individual guard/door status | BOOL array / individual BOOLs if exported | VERIFY FROM PILZ DATA MAP | MISSING UNTIL VERIFIED |
| Guard locking status | BOOLs if exported | VERIFY FROM PILZ DATA MAP | MISSING UNTIL VERIFIED |
| Safety-zone ready/not-ready | BOOLs if exported | VERIFY FROM PILZ DATA MAP | MISSING UNTIL VERIFIED |
| STO active / drive safety ready | BOOLs if exported | VERIFY FROM PILZ DATA MAP | MISSING UNTIL VERIFIED |
| Pneumatic dump/isolation active | BOOL if exported | VERIFY FROM PILZ DATA MAP | MISSING UNTIL VERIFIED |
| Safety pressure/feedback OK | BOOL if exported | VERIFY FROM PILZ DATA MAP | MISSING UNTIL VERIFIED |
| Standstill confirmed | BOOL if exported | VERIFY FROM PILZ DATA MAP | MISSING UNTIL VERIFIED |
| EDM/feedback discrepancy | BOOL(s) if exported | VERIFY FROM PILZ DATA MAP | MISSING UNTIL VERIFIED |
| Diagnostic code / status word | WORD/DWORD if exported | VERIFY FROM PILZ DATA MAP | MISSING UNTIL VERIFIED |

# 4. Siemens PLC Data Structure

Create a dedicated structured diagnostic interface, for example DB_PILZ_DIAG / UDT_PILZ_DIAG, following the existing project naming convention. Do not invent absolute I/Q addresses. First configure/verify the real PROFINET device and process-data mapping, then map the hardware data into logical diagnostic tags.

- Separate raw PROFINET process data from logical diagnostic tags.

- Provide communication-valid / data-valid indication.

- If PROFINET communication is lost, all dependent HMI diagnostic values must show INVALID/COMM FAULT, not a false normal/closed state.

- Keep diagnostic tags read-only from HMI unless an explicitly approved non-safety command is defined.

# 5. Pilz PROFINET Device Configuration

Audit the actual Pilz model, PROFINET capability, GSDML/device definition, device name, IP configuration and configured input/output byte length. If these are not present in the current TIA project, report them as MISSING and prepare the software structure without fabricating hardware configuration.

# 6. Safety / Pilz HMI Diagnostics Page

Continue the REV22 Safety / Pilz Diagnostics page, but source its detailed states from the verified Pilz→PLC PROFINET diagnostic structure.

- Overall Safety Ready / Not Ready / Fault.

- Pilz PROFINET communication status.

- Individual E-stop status when exported.

- Individual door/guard OPEN/CLOSED status when exported.

- LOCKED/UNLOCKED status only where a separate locking diagnostic is exported.

- Zone status.

- STO status.

- Pneumatic dump/isolation status.

- Safety pressure/feedback status.

- Standstill status.

- Reset-required state.

- Diagnostic/status code with plain-language interpretation where documented.

# 7. HMI Alarm Integration

- Generate standard HMI diagnostic alarms from verified Pilz non-safety PROFINET data.

- Examples: E-STOP ACTIVE, GUARD/DOOR OPEN, SAFETY RESET REQUIRED, STO ACTIVE, AIR DUMP ACTIVE, PILZ FAULT, PILZ PROFINET COMMUNICATION FAULT.

- Where individual device bits exist, alarm text shall identify the exact door/E-stop/zone.

- Each relevant alarm shall navigate to SAFETY / PILZ DIAGNOSTICS where supported.

- Alarm acknowledgement does not reset or override the Pilz safety function.

# 8. PLC → Pilz Non-Safety Data

If the engineering design also requires non-safety commands/status data from Siemens PLC to Pilz over PROFINET, implement only items explicitly defined by the Pilz engineer/data map. Do not infer or invent reset, mode, bypass or safety commands. Any safety reset architecture must remain consistent with the validated Pilz design.

# 9. Information Required From Pilz Engineer

☐ Exact Pilz controller/base-unit model and installed communication module, if applicable.

☐ PROFINET device/GSDML identification.

☐ Device name and IP addressing plan.

☐ Input byte/word map Pilz → Siemens.

☐ Output byte/word map Siemens → Pilz, if used.

☐ Bit-by-bit diagnostic definition.

☐ Door/E-stop/zone engineering IDs corresponding to each bit.

☐ Status/diagnostic word definitions.

# 10. Rob – Execution Instruction

Rob: REV23 clarifies REV22. The Pilz safety controller does NOT need to be recreated as Siemens safety logic. The confirmed architecture is that NON-SAFETY diagnostic data from Pilz is transferred to the Siemens PLC via PROFINET. Prepare and implement a dedicated Pilz diagnostic data structure in the Siemens PLC and use it as the source for the Safety / Pilz Diagnostics HMI page and Safety diagnostic alarms. Audit the actual Pilz PROFINET device configuration and data map before binding addresses. Do not invent GSDML, IP addresses, process-image offsets, byte lengths, individual door/E-stop channels, STO bits, air-dump bits or EDM feedback. If the detailed Pilz data map is not yet available, create the logical PLC/HMI structure and mark the physical PROFINET mapping MISSING/TBC. When the map is supplied, bind each verified bit/word so the HMI can identify the exact door, E-stop, zone or safety function causing the machine not-ready condition. On PROFINET communication loss, show COMMUNICATION FAULT / DATA INVALID rather than a false safe/closed condition. Safety authority remains entirely in Pilz; Siemens PLC/HMI uses these signals for diagnostics, alarms and non-safety sequencing only.

# 11. Acceptance / Close-Out

☐ Pilz→Siemens communication path identified as PROFINET.

☐ Actual Pilz PROFINET device configuration verified or explicitly reported missing.

☐ Raw PROFINET data separated from logical diagnostic tags.

☐ Detailed door/E-stop/zone status bound only to verified data-map bits.

☐ HMI Safety/Pilz page uses the verified diagnostic structure.

☐ Pilz communication loss creates a clear invalid/fault state.

☐ Safety alarms identify individual devices when the data map supports it.

☐ No safety function is transferred to or bypassed by the standard Siemens PLC/HMI.

☐ PLC/HMI compile with zero errors after final mapping.