ROB – REV21
FUNCTIONAL ALARM MATRIX

Caps • Bottles • Process • Utilities • Drives • CIP • Communications • Safety

# 1. Purpose / Implementation Rule

Implement a structured PLC/HMI alarm philosophy for the refurbished monoblock. Reuse existing alarms, tags, interlocks and native WinCC Unified Alarm Control wherever they already exist. Do not duplicate alarms and do not invent field feedbacks. This REV21 defines the functional alarm intent; Rob shall cross-reference the actual TIA project before binding.

# 2. Alarm Classes

- WARNING – operator attention required; machine may continue or reduce speed.

- CONTROLLED STOP – stop production sequence in a controlled manner; retain safe machine state.

- FAULT / TRIP – affected equipment or machine stops and requires fault clearance/reset.

- SAFETY – safety function has acted. Display diagnostic cause/status only; Safety PLC remains authoritative.

- COMMUNICATION / QUALITY – device/network data invalid or unavailable; must be visually distinct from a valid OFF/zero state.

# 3. Cap System Alarm Matrix

| ID | Alarm Text | Trigger / Source | Class | Machine Response | Operator Guidance |
| --- | --- | --- | --- | --- | --- |
| CAP-001 | Cap hopper low level | DI-060 / confirmed cap-low signal | WARNING | Continue; warning only until configured escalation | Refill cap hopper |
| CAP-002 | Cap channel request / low supply | DI-061 / cap channel request | WARNING | Continue or controlled slowdown per existing sequence | Check cap feed and refill |
| CAP-003 | Cap channel empty / caps unavailable | DI-062 / cap channel empty | CONTROLLED STOP | Prevent further uncapped bottle production; controlled stop/gate action | Restore caps; clear channel; acknowledge |
| CAP-004 | Cap not present at pick & place | DI-040 / cap present P&P false when expected | CONTROLLED STOP | Stop capping sequence / reject or inhibit cycle according to existing logic | Check cap chute, pick head and sensor |
| CAP-005 | Cap feed jam / abnormal cap flow | Existing jam/timeout logic if available | FAULT | Stop affected cap system | Clear jam; inspect chute/hopper |
| CAP-006 | Capper drive fault | Capper VFD/drive fault | FAULT | Stop capper and production sequence | Check drive diagnostic then reset |
| CAP-007 | Capper not ready | Capper permissive/ready missing | CONTROLLED STOP | Production start inhibited / controlled stop | Check capper permissives |

# 4. Bottle Flow / Conveyor Alarms

BOT-001 – Bottle shortage level 1: Source DI-031; WARNING. Response: Reduce/coordinate line speed. Action: Restore infeed.

BOT-002 – Bottle shortage level 2: Source DI-032; CONTROLLED STOP. Response: Gate/standby or controlled stop. Action: Restore bottle supply.

BOT-003 – Outfeed accumulation level 1: Source DI-033; WARNING. Response: Slow line. Action: Clear downstream accumulation.

BOT-004 – Outfeed accumulation level 2: Source DI-034; CONTROLLED STOP. Response: Gate/standby or controlled stop. Action: Clear outfeed.

BOT-005 – Bottle detection timeout / missing expected bottle: Source Existing sequence timeout; FAULT. Response: Stop affected sequence. Action: Check bottle path/sensor.

# 5. Pneumatic / Process / Product Alarms

| ID | Alarm | Source | Class | Response | Action |
| --- | --- | --- | --- | --- | --- |
| PROC-001 | Machine air pressure low / not OK | DI-010 / pressure signal | FAULT | Inhibit pneumatic motion / controlled stop | Restore air pressure |
| PROC-002 | Product pressure low | Confirmed product-pressure analog signal | WARNING/STOP by validated threshold | Warn or inhibit filling as defined | Check product supply/pump |
| PROC-003 | Product pressure high | Confirmed product-pressure analog signal | FAULT | Stop/inhibit product delivery | Check regulator/pump/valves |
| PROC-004 | Product level low | Confirmed level signal | WARNING | Warn / prepare product supply | Restore product |
| PROC-005 | Product level critically low | Confirmed level signal | CONTROLLED STOP | Inhibit filling / stop before dry running | Restore product |
| PROC-006 | Process temperature out of range | Confirmed temperature signal | WARNING/FAULT | Per validated process limits | Check utility/process |
| PROC-007 | Vacuum not achieved | M103 vacuum value / timeout | FAULT | Inhibit affected filling cycle | Check M103, leaks and valves |
| PROC-008 | Vacuum pump M103 fault | M103 drive/device fault | FAULT | Stop affected process | Open M103 diagnostics |
| PROC-009 | Product pump M102 fault | M102 drive/device fault | FAULT | Stop product delivery/filling | Open M102 diagnostics |

# 6. Drives / Motion / Machine Sequence

- DRV-001 – Main carousel/filler drive fault: FAULT; controlled machine stop; link to drive diagnostics.

- DRV-002 – Conveyor drive fault: FAULT; stop/coordinate upstream-downstream motion.

- DRV-003 – Drive not ready / communication unavailable: start inhibited; distinguish network fault from drive trip.

- DRV-004 – Encoder/synchronism fault: FAULT; stop synchronized motion; preserve deterministic encoder architecture.

- SEQ-001 – Machine sequence timeout: FAULT; identify step number and expected condition on HMI.

- SEQ-002 – Invalid machine state / conflicting mode request: FAULT; reject transition and show cause.

# 7. CIP Alarm Matrix

- CIP-001 – Customer CIP remote not ready when requested: inhibit phase transition / timeout alarm.

- CIP-002 – Customer CIP remote fault: abort CIP to defined safe state.

- CIP-003 – Requested medium not accepted/available within timeout: abort or hold according to validated handshake.

- CIP-004 – Discharge not completed within timeout: hold sequence and alarm.

- CIP-005 – Conflicting CIP media requests: PLC logic fault; immediately clear conflicting requests and prevent continuation.

- CIP-006 – CIP phase timeout: hold/abort and display active step.

- CIP-007 – CIP sequence aborted by operator: controlled abort status; not treated as an unexplained equipment trip.

- CIP-008 – AL104 communication fault: invalidate customer interface and prevent new CIP requests.

- CIP-009 – CIP complete awaiting operator acknowledgement: INFORMATION / status, not a fault.

# 8. IO-Link / Network / SMC Communications

- COM-100…104 – AL100/AL101/AL102/AL103/AL104 master communication fault. Alarm must link directly to the relevant master diagnostic page.

- COM-110 – IO-Link port device missing where a commissioned device is expected.

- COM-111 – IO-Link port bad quality / device diagnostic active.

- COM-120 – Anybus ABC3113-A PROFINET communication fault.

- COM-121 – Anybus EtherCAT side fault.

- COM-122 – SMC EX260-SEC1 communication/device fault.

- COM-123 – SMC output data invalid. Pneumatic commands shall fail to the validated safe behavior.

# 9. Safety Diagnostics

Safety alarms are diagnostic representations of the Safety system only. Rob must not recreate or bypass Safety logic in the standard PLC/HMI.

- SAFE-001 – Emergency stop active.

- SAFE-002 – Guard/door safety circuit open.

- SAFE-003 – Safety controller / safety communication fault.

- SAFE-004 – Safety reset required / conditions not satisfied.

- SAFE-005 – Safety actuator/contactor feedback discrepancy where provided by the safety design.

# 10. HMI Alarm Behaviour

- Preserve the native WinCC Unified Alarm Control.

- Show alarm ID, timestamp, class/severity, plain-language message and equipment/device ID.

- Add operator guidance text where supported.

- Alarm acknowledgement must not automatically reset the underlying machine fault.

- Reset is permitted only when the PLC fault condition and required permissives are cleared.

- Provide GO TO DIAGNOSTICS for AL100–AL104, M102, M103, SMC/Anybus and other supported devices.

- Use red for active fault/trip, amber for warning, and a distinct communication/bad-quality indication.

- Do not use normal-state blinking.

# 11. Alarm Flood / Delay / Debounce Rules

- Use existing validated input debounce/filtering where present.

- Apply sensible on-delay to shortage/accumulation warnings where required to avoid nuisance alarms.

- Use explicit sequence timeouts rather than raw instantaneous alarms for conditions that require time to establish.

- Suppress consequential alarms where the primary cause already explains the stopped state, when supported by existing architecture.

- Do not hide safety alarms or critical equipment trips.

# 12. Rob Implementation Command

Rob: implement REV21 as the functional alarm matrix. Audit the existing TIA PLC/HMI alarm configuration first and reuse existing alarms, tags, events, texts, reset logic and native WinCC Unified Alarm Control. Add the missing functional alarms for the cap system, including Cap Hopper Low, Cap Channel Request/Low Supply, Cap Channel Empty/Caps Unavailable, Cap Not Present at Pick & Place, Cap Feed Jam/Timeout, Capper Drive Fault and Capper Not Ready. Implement bottle shortage/accumulation alarms from DI-031…DI-034, pneumatic/process alarms, M102/M103 alarms, drive/sequence alarms, CIP handshake/timeout alarms, AL100–AL104 IO-Link communication/device alarms, Anybus/SMC communication alarms and Safety diagnostic messages. Do not invent field signals. Where a requested alarm has no proven source, mark it MISSING/AMBIGUOUS and report it rather than creating a false binding. Each alarm shall have a defined class, machine response, operator guidance, acknowledgement/reset behavior and diagnostic navigation where supported. Alarm acknowledgement must never bypass the underlying PLC/Safety condition. Compile PLC/HMI and provide an alarm matrix report showing IMPLEMENTED / EXISTING / MISSING / AMBIGUOUS for every REV21 alarm.

# 13. Acceptance Checklist

☐ Cap shortage/empty/not-present conditions are represented.

☐ Warnings and machine-stop faults are differentiated.

☐ Every implemented alarm has a proven source tag/condition.

☐ Alarm acknowledgement and machine reset are separate where required.

☐ IO-Link alarms navigate to AL100–AL104 diagnostics.

☐ CIP alarms identify current phase/handshake cause.

☐ Communication bad quality is not shown as a normal OFF condition.

☐ Safety diagnostics do not modify Safety logic.

☐ PLC compile = 0 errors.

☐ HMI compile = 0 errors.

☐ Final alarm report identifies all unresolved sources.