ROB – REV20
AL1403 MASTER PORT ALLOCATION – PLC / HMI / WIRING

REV20 supplements REV18 with a frozen construction allocation for AL100–AL104.

# 1. Construction Rule

Use the port allocation below as the MASTER construction convention. Rob shall build PLC structures, HMI diagnostics, electrical drawing references and commissioning tables to the same ALxxx/Px allocation. The workshop shall wire field devices to these ports. Any later physical change must be changed in all four places: wiring drawing, PLC mapping, HMI diagnostics and this master table.

Important source boundary: the existing official project documentation confirms nine legacy IO-Link digital functions and requires master spare capacity, but it does not assign those functions to specific AL1403 ports. The port numbers below are therefore a NEW REV20 design allocation, intentionally frozen now for construction.

# 2. Master Functional Allocation

- AL100 – Bottle Infeed / Outfeed Detection

- AL101 – Cap Handling / Pneumatic Utility

- AL102 – Process Instrumentation / Product System

- AL103 – Expansion / Additional Smart Sensors

- AL104 – Dedicated Customer CIP Interface

## 3. AL100 – Bottle Infeed / Outfeed Detection

| Port | Device ID | Function / HMI Description | Device / Type | PLC Logical Tag | Mode | Status |
| --- | --- | --- | --- | --- | --- | --- |
| P1 | DI-031 | Bottle Shortage 1 – line slowdown | Field sensor; final physical model per build | b_BottleShortage_1 | DI / SIO or IO-Link as installed | FROZEN PORT |
| P2 | DI-032 | Bottle Shortage 2 – gate/standby | Field sensor; final physical model per build | b_BottleShortage_2 | DI / SIO or IO-Link as installed | FROZEN PORT |
| P3 | DI-033 | Outfeed Accumulation 1 – slowdown | Field sensor; final physical model per build | b_Accumulation_1 | DI / SIO or IO-Link as installed | FROZEN PORT |
| P4 | DI-034 | Outfeed Accumulation 2 – gate/standby | Field sensor; final physical model per build | b_Accumulation_2 | DI / SIO or IO-Link as installed | FROZEN PORT |
| P5 | SPARE-A100-05 | Future bottle sensor | — | — | Spare | SPARE |
| P6 | SPARE-A100-06 | Future bottle sensor | — | — | Spare | SPARE |
| P7 | SPARE-A100-07 | Future expansion | — | — | Spare | SPARE |
| P8 | SPARE-A100-08 | Future expansion | — | — | Spare | SPARE |

## 4. AL101 – Cap Handling / Pneumatic Utility

| Port | Device ID | Function / HMI Description | Device / Type | PLC Logical Tag | Mode | Status |
| --- | --- | --- | --- | --- | --- | --- |
| P1 | DI-040 | Cap Present – Pick & Place | Field sensor; final physical model per build | b_CapPresent_PP | DI / SIO or IO-Link as installed | FROZEN PORT |
| P2 | DI-060 | Cap Hopper Low Level | Field sensor; final physical model per build | b_CapHopper_Low | DI / SIO or IO-Link as installed | FROZEN PORT |
| P3 | DI-061 | Cap Channel Request | Field sensor; final physical model per build | b_CapChannel_Req | DI / SIO or IO-Link as installed | FROZEN PORT |
| P4 | DI-062 | Cap Channel Empty | Field sensor; final physical model per build | b_CapChannel_Empty | DI / SIO or IO-Link as installed | FROZEN PORT |
| P5 | DI-010 | Machine Air Pressure OK | Pressure switch / sensor | b_AirPressureOK | DI / IO-Link if supported | FROZEN PORT |
| P6 | SPARE-A101-06 | Cap / pneumatic expansion | — | — | Spare | SPARE |
| P7 | SPARE-A101-07 | Cap / pneumatic expansion | — | — | Spare | SPARE |
| P8 | SPARE-A101-08 | Future expansion | — | — | Spare | SPARE |

## 5. AL102 – Process Instrumentation / Product System

| Port | Device ID | Function / HMI Description | Device / Type | PLC Logical Tag | Mode | Status |
| --- | --- | --- | --- | --- | --- | --- |
| P1 | PT-PRODUCT | Product Pressure | IFM PN7094 candidate from purchased BOM | ai_ProductPressure | IO-Link | ASSIGN DEVICE AT BUILD |
| P2 | PT-AIR | Pneumatic / Process Pressure Diagnostic | IFM PN7094 candidate | ai_AirPressure | IO-Link | ASSIGN DEVICE AT BUILD |
| P3 | PT-CIP | CIP / Service Pressure Diagnostic | IFM PN7094 candidate | ai_CIPPressure | IO-Link | ASSIGN DEVICE AT BUILD |
| P4 | PT-SPARE | Spare Pressure Measurement | IFM PN7094 candidate | ai_SparePressure | IO-Link | SPARE INSTRUMENT |
| P5 | LT-PRODUCT | Product Level / Continuous Level | IFM LR2050 candidate | ai_ProductLevel | IO-Link | ASSIGN DEVICE AT BUILD |
| P6 | LT-CIP | CIP / Process Level | IFM LR2050 candidate | ai_CIPLevel | IO-Link | ASSIGN DEVICE AT BUILD |
| P7 | TT-PROCESS | Process Temperature | IFM TA2405 candidate | ai_ProcessTemperature | IO-Link | ASSIGN DEVICE AT BUILD |
| P8 | SPARE-A102-08 | Future process instrument | — | — | Spare | SPARE |

## 6. AL103 – Additional Smart Sensors / Expansion

| Port | Device ID | Function / HMI Description | Device / Type | PLC Logical Tag | Mode | Status |
| --- | --- | --- | --- | --- | --- | --- |
| P1 | SEN-103-01 | Additional machine sensor 1 | IFM purchased sensor – assign during mechanical installation | io_AL103_P1 | IO-Link/SIO | RESERVED |
| P2 | SEN-103-02 | Additional machine sensor 2 | IFM purchased sensor – assign during mechanical installation | io_AL103_P2 | IO-Link/SIO | RESERVED |
| P3 | SEN-103-03 | Additional machine sensor 3 | IFM purchased sensor – assign during mechanical installation | io_AL103_P3 | IO-Link/SIO | RESERVED |
| P4 | SEN-103-04 | Additional machine sensor 4 | IFM purchased sensor – assign during mechanical installation | io_AL103_P4 | IO-Link/SIO | RESERVED |
| P5 | SPARE-A103-05 | Future smart sensor | — | — | Spare | SPARE |
| P6 | SPARE-A103-06 | Future smart sensor | — | — | Spare | SPARE |
| P7 | SPARE-A103-07 | Future smart sensor | — | — | Spare | SPARE |
| P8 | SPARE-A103-08 | Future smart sensor | — | — | Spare | SPARE |

## 7. AL104 – Dedicated Customer CIP Interface

| Port | Device ID | Function / HMI Description | Device / Type | PLC Logical Tag | Mode | Status |
| --- | --- | --- | --- | --- | --- | --- |
| P1 | CIP-DO-01 | Cold Water Request | Customer interface output | CIP_REQ_COLD_WATER | Digital output | FROZEN FUNCTION |
| P2 | CIP-DO-02 | Hot Water Request | Customer interface output | CIP_REQ_HOT_WATER | Digital output | FROZEN FUNCTION |
| P3 | CIP-DO-03 | Acid / Chemical Request | Customer interface output | CIP_REQ_ACID | Digital output | FROZEN FUNCTION |
| P4 | CIP-DO-04 | Citra / Neutralizer Request | Customer interface output | CIP_REQ_CITRA | Digital output | FROZEN FUNCTION |
| P5 | CIP-DO-05 | Discharge Request | Customer interface output | CIP_REQ_DISCHARGE | Digital output | FROZEN FUNCTION |
| P6 | PROD-DO-01 | Production / Product Request | Customer interface output | PRODUCTION_REQ_PRODUCT | Digital output | FROZEN FUNCTION |
| P7 | CIP-DI-01 | Customer CIP Ready / Accepted | Customer feedback – final electrical signal to confirm | CIP_REMOTE_READY | Digital input | RESERVED FEEDBACK |
| P8 | CIP-DI-02 | Customer CIP Fault / Busy | Customer feedback – final allocation to confirm | CIP_REMOTE_FAULT | Digital input | RESERVED FEEDBACK |

# 8. Purchased IFM Device Pool – Integration Rule

The purchased IFM list includes AL1403 masters and device families including IFS240, O5D100, O6P301, KI6000, PN7094, LR2050, TA2405 and RO3101. Do not arbitrarily substitute a product code into a machine function unless its physical application is confirmed during assembly. REV20 freezes WHERE the function connects; the exact sensor model for generic detection points may be finalized when the sensor is physically mounted.

RO3101 must not be moved onto an AL1403 merely because it supports IO-Link if it is being used as the machine motion encoder. The existing machine architecture uses the encoder A/B/Z with the deterministic Siemens counting/synchronism path; preserve that architecture unless separately redesigned.

# 9. HMI Requirements for Each Master Page

- Replace generic TBC rows with the REV20 Device ID and Function Description.

- Show P1–P8 exactly in the same order as this document.

- Show device model when physically confirmed.

- Show live state/value and engineering unit.

- Show quality, port fault and master communication status.

- SPARE ports must visibly read SPARE, not TBC.

- Reserved-but-not-yet-installed devices must read RESERVED / NOT INSTALLED.

- AL104 shall show the CIP command/feedback semantic names, not generic I/O.

# 10. PLC Requirements

- Create/maintain one structured mapping per master AL100–AL104.

- Logical machine tags remain independent from physical PROFINET process-image addresses.

- Map physical AL1403 port data to the logical tags only after TIA hardware/device configuration is verified.

- Do not invent PROFINET addresses or process-image offsets.

- Add diagnostics for master communication, port quality and device fault.

- Use the same Device IDs and descriptions as HMI and electrical drawings.

# 11. Electrical / Workshop Wiring Rule

The workshop shall label each master AL100–AL104 and each field cable with the destination port. Example: DI-031 → AL100/P1. Electrical drawings shall reproduce this exact allocation. If a physical installation constraint forces a change, STOP and update the master allocation before wiring the device elsewhere.

# 12. AL104 Electrical Caution

Before directly connecting AL104 outputs to the customer's CIP panel, confirm the customer's required electrical interface. If the customer requires dry/potential-free contacts, use suitable interposing relays/interface isolation. Do not assume that an AL1403 digital output can be connected directly until the customer's input specification is confirmed.

# 13. Rob – Immediate Implementation Command

Rob: implement REV20 now as the master AL1403 construction allocation. Update the existing AL100–AL104 IO-Link diagnostic pages so each port P1–P8 uses the Device ID, functional description and status defined in REV20. Use the same allocation in PLC mapping structures and electrical documentation. Treat AL100/P1–P4 as bottle-flow functions DI-031…DI-034; AL101/P1–P5 as cap-handling functions DI-040, DI-060, DI-061, DI-062 and machine air pressure DI-010; AL102 as process instrumentation allocation; AL103 as additional smart-sensor/expansion allocation; and AL104 as the dedicated CIP interface with Cold Water, Hot Water, Acid/Chemical, Citra, Discharge, Product and customer feedback channels. Do not invent physical PROFINET addresses or generic sensor product codes where the physical device application is not yet confirmed. Mark unused ports SPARE and planned but not installed points RESERVED / NOT INSTALLED. Preserve the deterministic machine encoder architecture and do not move the machine encoder to AL1403 without a separate approved redesign. Compile PLC/HMI after changes and provide the updated five master screenshots plus the mapping/open-item report.

# 14. Acceptance

☐ AL100–AL104 pages no longer show generic Functional Assignment TBC for frozen REV20 functions.

☐ Every P1–P8 row has a Device ID or explicit SPARE/RESERVED status.

☐ PLC/HMI/electrical naming matches exactly.

☐ AL104 CIP functions match the CIP sequence specification.

☐ No guessed physical network/process-image addresses.

☐ PLC compile = 0 errors; HMI compile = 0 errors.